# EchoROI: Scan-sector Segmentation and De-identification for Echocardiography

10 March 2026

Unrefereed technical manuscript; retained as material for a Biomedical Engineering doctoral thesis chapter at Stellenbosch University.

## Author affiliations

- Kamlin Ekambaram (corresponding author): Institute for Biomedical Engineering, Faculty of Engineering, Stellenbosch University, South Africa. ORCID: 0000-0002-1366-8451.

- Anurag Arnab: Independent Researcher.

- Philip Herbst: Division of Cardiology, Department of Medicine, Faculty of Medicine and Health Sciences, Stellenbosch University and Tygerberg Hospital, Cape Town, South Africa.

- Rensu Theart: Department of Electrical Engineering, Faculty of Engineering, Stellenbosch University, South Africa.

# Summary

Echocardiography is the most widely used cardiac imaging modality, but secondary analysis of echo videos is often limited by protected health information (PHI), electrocardiographic traces, measurement overlays, and vendor-specific graphics burned directly into pixel data (Madani et al. 2018). EchoROI is an open-source Python package that segments the fan-shaped ultrasound scan sector and masks non-diagnostic image content using a U-Net-based model (Ronneberger et al. 2015). The resulting outputs preserve the native scan-sector geometry while removing surrounding display content, enabling preprocessing for machine learning, privacy-aware sharing of teaching examples, and more consistent inputs for downstream analysis. The authors’ reference model targets the fan-shaped scan sector produced by phased-array cardiac probes and may generalise to similar curvilinear sector geometries, but is unlikely to segment the rectangular field of view of linear-array probes without fine-tuning or retraining on appropriate annotations.

EchoROI is distributed as an installable package via PyPI (pip install echoroi) and provides a command-line interface and a Python API. The repository includes training and evaluation workflows, Keras and ONNX support, export utilities, notebooks, aggregate reference-run records, and tests, but not pretrained weights or source datasets. Users must train locally or otherwise obtain a checkpoint lawfully before applying the inference workflows.

This release includes neither pretrained weights nor source datasets. Researchers must obtain source datasets independently from their providers. Requests for scan-sector annotations should be directed to the EchoROI authors; availability is assessed for each dataset under the applicable agreements and institutional permissions.

Figure 1. EchoROI preprocessing workflow. (a) Raw frame, with examples of burned-in overlays targeted for removal highlighted in red; (b) predicted scan-sector mask; (c) sector-masked output, where overlays outside the scan sector are removed but a partial ECG trace and partial scale bar are retained where they traverse the scan sector; and (d) ROI crop.

# Motivation

Echocardiography files commonly contain identifiers and vendor overlays rendered directly into image pixels. In research settings these artifacts can restrict data sharing and introduce shortcut signals for computer-vision models (Panhuis et al. 2014). Large collections such as MIMIC-IV-ECHO (Gow et al. 2023) therefore require preprocessing before they can be used safely for model development or shared for secondary analysis. Manual removal is feasible for small studies but becomes impractical at modern dataset scale. In prior work, the authors manually segmented and cropped approximately 1,000 apical four-chamber studies from MIMIC-IV-ECHO using LabelMe (Ekambaram et al. 2026; Russell et al. 2008), which motivated the development of a reusable automated workflow.

EchoROI was developed for researchers who need a scriptable and adaptable way to standardise echocardiography inputs across heterogeneous sources. Rather than relying on fixed crop templates, the authors’ reference model learned the sector boundary directly and uses the predicted mask to remove non-diagnostic content outside the scan sector. This supports privacy-aware preprocessing, multi-source dataset harmonisation, and workflows where background overlays would otherwise consume model capacity or bias evaluation. The package targets users preparing echocardiography datasets for machine learning, benchmarking, or educational reuse, especially when acquisition layouts vary across vendors, probe settings, or sites.

# Related Work

Existing approaches address related needs but not the full workflow targeted by EchoROI. OCR-based de-identification pipelines remove text from ultrasound files but do not explicitly recover the scan sector and may struggle with graphical overlays or variable vendor layouts (Monteiro et al. 2017). Heuristic cleaning tools such as PyLogik can work well on stable layouts, but layout-specific rules may require retuning when fan angle, depth, zoom, or surrounding display content changes (Kline et al. 2023). General medical imaging toolboxes such as ivadomed support segmentation model development across modalities (Gros et al. 2021), but they do not provide echocardiography-specific sector masking, representative-frame cine preprocessing, or de-identification-oriented outputs out of the box. Public datasets such as EchoNet-Dynamic also distribute pre-cropped videos (Ouyang et al. 2020), which simplifies downstream modelling but removes the original scan-sector geometry and assumes consistent cropping upstream.

EchoROI therefore occupies a distinct niche as reusable research software for learned scan-sector segmentation in heterogeneous echocardiography data. Its main contribution is not a new segmentation architecture, but a packaged workflow that combines inference with a user-supplied checkpoint, training, evaluation, and deployment-friendly export around the practical problem of sector masking and de-identification.

# Software Design

EchoROI is packaged as a modular Python library with components for preprocessing, model definition, training, inference, evaluation, and export. The command-line interface exposes the main workflows for prediction, fine-tuning, evaluation, and benchmarking, while the Python API supports integration into larger research pipelines. The core operation is mask prediction: the model segments the scan sector and outputs a binary mask for each input. When the optional --deidentify flag is set, the predicted mask is applied to zero out all pixels outside the scan sector, producing a sector-masked output in a single step. This operation does not remove identifiers or overlays within the predicted ROI and does not itself de-identify DICOM metadata. With a locally trained or otherwise lawfully obtained checkpoint at path/to/model.keras, a typical workflow is:

echoroi predict \
  --model-path path/to/model.keras \
  --input video_frames/ \
  --output clean/ \
  --deidentify

For echocardiography clips with fixed sector geometry, the DICOM preprocessing notebook selects a representative frame using grayscale Shannon entropy (Shannon 1948) and applies the predicted sector mask across the sequence. This notebook workflow reduces repeated inference and helps avoid blank or transition frames. The image-prediction CLI shown above operates on supplied image files; representative-frame selection is not a CLI default. Without --deidentify, only the predicted binary mask is saved, allowing users to apply it in custom downstream workflows or inspect residual traces such as ECG that may cross the sector boundary. The model performs sector segmentation only and does not attempt OCR or explicit overlay extraction.

The reference model uses a standard U-Net (Ronneberger et al. 2015) for binary scan-sector segmentation, adapted to this application with 256×256 grayscale inputs, same-padding convolutions to preserve sector geometry, and dropout regularisation (Srivastava et al. 2014) to reduce overfitting on a small heterogeneous annotation set. Training uses a composite loss that sums binary cross-entropy, Dice loss (Milletari et al. 2016), and a total-variation (TV) regularisation term (Rudin et al. 1992). The Dice component handles foreground/background class imbalance, while the TV term penalises high-frequency mask gradients and encourages the smooth, fan-shaped sector boundaries characteristic of phased-array ultrasound probes. The train/validation split (80/20) is stratified by source dataset so that every contributing collection is proportionally represented. The TensorFlow/Keras implementation prioritises robust segmentation and modest hardware requirements, and supports ONNX export for deployment outside TensorFlow.

The documented annotation collection contains 1,355 frame-mask pairs from public and institutional echocardiography datasets, including MIMIC-IV-ECHO (Gow et al. 2023), EchoNet-Dynamic (Ouyang et al. 2020), EchoNet-Paediatric (Reddy et al. 2023), CACTUS (Elmekki et al. 2025), EchoCP (Wang et al. 2021), CardiacUDC (Yang et al. 2023), HMC-QU (Degerli et al. 2024), and a small consented institutional set. Sector masks were created in LabelMe (Russell et al. 2008) by outlining the visible scan sector while excluding padding and display graphics. One representative frame per cine loop was used because sector geometry is typically static within a clip; a per-source breakdown is provided in the repository documentation. This collection inventory must be distinguished from the saved reference-run records, which report 1,206 loaded pairs (964 training and 242 validation), batch size 16, 50 epochs, and learning rate 10−4. The saved run’s source code selects checkpoints by validation IoU; batch size 8 is the current command-line default, not the recorded run setting. There is no saved per-file manifest establishing which collection members were used or excluded, so the 149-pair difference cannot be attributed to a particular source from these records. The detailed reconciliation of collection counts and saved run records is provided in training_results/README.md. A collection count alone does not establish the membership of a training run.

# Validation and Reuse

The documented workflow is to obtain source datasets independently from their providers; request any available sector masks from the authors; use the authorised manifest and decoding recipe to match each mask to its original native decoded frame; train and evaluate locally using the retraining instructions; and optionally export the trained checkpoint to ONNX. Neither the source datasets nor pretrained weights are included. Annotation availability depends on provider and institutional permissions; the private subset may be unavailable.

The authors’ saved reference-model evaluation reports the following results on the 242-sample validation split described in the evaluation notebook:

| Metric | Value |
| --- | --- |
| Dice coefficient | 0.9884 |
| IoU | 0.9770 |
| Pixel accuracy | 0.9907 |
| Sensitivity | 0.9891 |
| Specificity | 0.9918 |

These figures come from the validation split used for model selection, so they should be interpreted as software validation rather than a definitive external benchmark. These results support the feasibility of sector masking on the recorded validation data. Performance on new sites, scanner layouts, and acquisition conditions requires separate assessment. Implementation-specific benchmarking examples are provided in the repository notebooks.

The values above are thresholded, pooled-pixel metrics from training_results/metrics.json. The training log instead records best soft validation Dice 0.98726451397 and IoU 0.97487163544, both at zero-based epoch 47. The complete original training membership is unresolved and the private subset may be unavailable, so retraining does not guarantee the identical checkpoint or reported metrics. No released per-file split or evaluation-checkpoint hash establishes exact reproduction. These are results of the documented reference experiment, not an external test benchmark; see the reference-run records for the saved configuration and metric definitions.

The repository supports reuse of the software once researchers have obtained authorised data, annotations, and a local checkpoint. It includes example notebooks, aggregate evaluation metrics, training logs, and tests. The access and retraining documentation describes a pathway for new training runs, not a guarantee of exact reproduction of the reference checkpoint. Source code and aggregate records are available at https://github.com/Kamlin-MD/UNET-Echocardiography-ROI-segmentation. The included evaluation command and DICOM-to-NPZ notebook support end-to-end preprocessing pipelines in which clips are loaded, optionally resampled, masked using EchoROI, cropped using the predicted sector geometry, and saved for downstream modelling using a user-supplied model. This combination of software, documented reference results, and adaptable workflows supports EchoROI’s use as a preprocessing component for privacy-aware dataset preparation and standardised input generation in echocardiography research.

EchoROI should nonetheless be used as a preprocessing aid rather than a guarantee of complete PHI removal. On still frames, clinicians may place measurement callipers, annotations, or text labels at arbitrary positions within the scan sector; because this placement is user-dependent, such overlays can include patient identifiers or clinical notes that the sector mask cannot remove. Cine clips are typically saved without user-placed in-sector annotations, but machine-generated temporal overlays — such as ECG or respiratory traces — may traverse the sector boundary and therefore remain after masking, as illustrated in Figure 1. These traces are unlikely to contain PHI, but human review and local governance procedures remain necessary before external data sharing.

# AI Usage Disclosure

GitHub Copilot in Visual Studio Code was used for limited repository maintenance and manuscript revision. All AI-assisted changes were reviewed, edited, and validated by the authors, who take responsibility for the final software and manuscript.

# Acknowledgements

This work was supported by the University of Stellenbosch Institute of Biomedical Engineering and in part by the National Research Foundation of South Africa (Grant Number: TTK240321210363). The manual A4C segmentation effort described in (Ekambaram et al. 2026) directly motivated the development of EchoROI. We gratefully acknowledge the providers of the datasets used for training and evaluation, including PhysioNet and Beth Israel Deaconess Medical Center for MIMIC-IV-ECHO (Gow et al. 2023; Goldberger et al. 2000), the Stanford AIMI Center for EchoNet-Dynamic (Ouyang et al. 2020) and EchoNet-Paediatric (Reddy et al. 2023), and the authors of CACTUS (Elmekki et al. 2025), EchoCP (Wang et al. 2021), CardiacUDC (Yang et al. 2023), and HMC-QU (Degerli et al. 2024). EchoROI is built on TensorFlow/Keras, OpenCV, and the Python scientific computing ecosystem.

# References

Degerli, Aysen, Serkan Kiranyaz, Tahir Hamid, Rashid Mazhar, and Moncef Gabbouj. 2024. “Early Myocardial Infarction Detection over Multi-View Echocardiography.” Biomedical Signal Processing and Control 87. https://doi.org/10.1016/j.bspc.2023.105448.

Ekambaram, Kamlin, Anurag Arnab, Philip Herbst, and Rensu Theart. 2026. “MIMIC-IV-ECHO-Ext-LVVOLUMES-A4C-ROI: Annotated Subset of Apical Four-Chamber Echocardiography for PoCUS-Style LV Volume and Function Analysis.” PhysioNet, ahead of print, February. https://doi.org/10.13026/713s-z339.

Elmekki, Hanae, Ahmed Alagha, Hani Sami, et al. 2025. “CACTUS: An Open Dataset and Framework for Automated Cardiac Assessment and Classification of Ultrasound Images Using Deep Transfer Learning.” Computers in Biology and Medicine 190: 110003. https://doi.org/10.1016/j.compbiomed.2025.110003.

Goldberger, Ary L., Luı́s A. Nunes Amaral, Leon Glass, et al. 2000. “PhysioBank, PhysioToolkit, and PhysioNet: Components of a New Research Resource for Complex Physiologic Signals.” Circulation 101 (23): e215–20. https://doi.org/10.1161/01.CIR.101.23.e215.

Gow, Brian, Tom Pollard, Larry A. Nathanson, et al. 2023. MIMIC-IV-Echo: Echocardiogram Matched Subset. PhysioNet. https://doi.org/10.13026/ef48-v217.

Gros, Charley, Andreanne Lemay, Olivier Vincent, et al. 2021. “Ivadomed: A Medical Imaging Deep Learning Toolbox.” Journal of Open Source Software 6 (58): 2868. https://doi.org/10.21105/joss.02868.

Kline, Adrienne, Shiyuan Zhou, Chi-Chun Liu, and William K. Thompson. 2023. Medical Image de-Identification, Cleaning and Compression Using PyLogik. https://doi.org/10.48550/arXiv.2302.13905.

Madani, Ali, Ramy Arnaout, Mohammad Mofrad, and Rima Arnaout. 2018. “Fast and Accurate View Classification of Echocardiograms Using Deep Learning.” Npj Digital Medicine 1 (1): 6. https://doi.org/10.1038/s41746-017-0013-1.

Milletari, Fausto, Nassir Navab, and Seyed-Ahmad Ahmadi. 2016. “V-Net: Fully Convolutional Neural Networks for Volumetric Medical Image Segmentation.” Proceedings of the Fourth International Conference on 3D Vision (3DV), 565–71. https://doi.org/10.1109/3DV.2016.79.

Monteiro, Erin Eriksson, Javier Andreu-Perez, Marcus Larsson, Krister Boman, Panagiotis Papapetrou, and Theo Kyriacou. 2017. “A de-Identification Pipeline for Ultrasound Medical Images in DICOM Format.” Journal of Medical Systems 41 (6): 97. https://doi.org/10.1007/s10916-017-0747-6.

Ouyang, David, Bryan He, Amirata Ghorbani, et al. 2020. “Video-Based AI for Beat-to-Beat Assessment of Cardiac Function.” Nature 580 (7802): 252–56. https://doi.org/10.1038/s41586-020-2145-8.

Panhuis, Willem G van, Proma Paul, Claudia Emerson, et al. 2014. “A Systematic Review of Barriers to Data Sharing in Public Health.” BMC Public Health 14 (1): 1144. https://doi.org/10.1186/1471-2458-14-1144.

Reddy, Charitha, Leo Lopez, David Ouyang, James Y. Zou, and Bryan He. 2023. “Video-Based Deep Learning for Automated Assessment of Left Ventricular Ejection Fraction in Pediatric Patients.” Journal of the American Society of Echocardiography 36 (5): 482–89. https://doi.org/10.1016/j.echo.2023.01.015.

Ronneberger, Olaf, Philipp Fischer, and Thomas Brox. 2015. “U-Net: Convolutional Networks for Biomedical Image Segmentation.” Medical Image Computing and Computer-Assisted Intervention (MICCAI), 234–41. https://doi.org/10.1007/978-3-319-24574-4_28.

Rudin, Leonid I., Stanley Osher, and Emad Fatemi. 1992. “Nonlinear Total Variation Based Noise Removal Algorithms.” Physica D: Nonlinear Phenomena 60 (1–4): 259–68. https://doi.org/10.1016/0167-2789(92)90242-F.

Russell, Bryan C., Antonio Torralba, Kevin P. Murphy, and William T. Freeman. 2008. “LabelMe: A Database and Web-Based Tool for Image Annotation.” International Journal of Computer Vision 77 (1-3): 157–73. https://doi.org/10.1007/s11263-007-0090-8.

Shannon, Claude E. 1948. “A Mathematical Theory of Communication.” Bell System Technical Journal 27 (3): 379–423. https://doi.org/10.1002/j.1538-7305.1948.tb01338.x.

Srivastava, Nitish, Geoffrey Hinton, Alex Krizhevsky, Ilya Sutskever, and Ruslan Salakhutdinov. 2014. “Dropout: A Simple Way to Prevent Neural Networks from Overfitting.” Journal of Machine Learning Research 15 (56): 1929–58. https://doi.org/10.5555/2627435.2670313.

Wang, Tianchen et al. 2021. “EchoCP: An Echocardiography Dataset in Contrast Transthoracic Echocardiography for Patent Foramen Ovale Diagnosis.” Medical Image Computing and Computer Assisted Intervention (MICCAI 2021), Lecture notes in computer science, vol. 12906. https://doi.org/10.1007/978-3-030-87231-1_49.

Yang, Jiewen, Xinpeng Ding, Ziyang Zheng, Xiaowei Xu, and Xiaomeng Li. 2023. GraphEcho: Graph-Driven Unsupervised Domain Adaptation for Echocardiogram Video Segmentation. https://doi.org/10.48550/arXiv.2309.11145.